Bangladesh University of Engineering and Technology (BUET)

Department of Computer Science and Engineering (CSE)

CSE 210	Digital Electronics and Pulse Techniques Sessional

Experiment# 6: Study of diode clipping circuits

CKT diagram

| Fig. 1: Simple clipping circuit | Fig. 2: Two-level clipping circuit |
| --- | --- |

Apparatus:

- Diode     1N4007      2 pcs

- Resistor 100KΩ        1 pc

Procedure:

- Fig. 1:

  - Observe the output for Vi=10V p-p 50 Hz sine wave and VR=3V.

  - Reverse the diode and observe its effect.

  - Reverse VR and observe its effect.

  - Reverse both the diode and VR, and observe its effect.

- Fig. 2:

  - Observe the output for Vi=10V p-p 50 Hz sine wave and VR1=3V and VR2=4V.

  - Reverse VR1 and observe its effect.

Question:

- In Fig. 2, what would happen for the following:

  - Reverse only VR2.

  - Reverse both VR1 and VR2.

- What will be the transfer characteristics graphs for all cases you have experimented with and you have considered in the previous question?

Report:

Report should cover the following points:

- Objective

- Circuit diagram and input-output wave shapes

- Answer to the questions

- Discussion of the findings

- Applications of your study