Bangladesh University of Engineering and Technology

Department of Computer Science and Engineering

CSE 316N

Microprocessors and Microcontrollers Sessional

Experiment 1A

Basic data transmission and reception through 8255.

GOAL:

Understanding the way CPU sends and receives data from other devices to control them.

EXPERIMENTAL TOOLS AND MATERIALS: MTS-88.C and I/O BOARD -01

EXPLANATION OF PRINCIPLES:

8255 mode 0 will be used to do input-output. MTS-88.C system board has an 8255 Programmable Peripheral Interfacing (PPI) chip connected with the microprocessor 8088 at address 000100xx b.  DIP switches of the I/O Board are connected with Port A and LEDs are connected with Port B of 8255.

PROCEDURE:

- Connect I/O Board-01 with the system board by 40 pins flat cable.

- Configure 8255 Port A as input port and Port B as output port.

- Write an 8088 assembly program that will read the status of the DIP switches, manipulate the info as instructed during lab class by the lab instructors and glow the LEDs accordingly.

### CONTROL WORD OF 8255:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode Set Bit 1 | Group A Mode Selection 00=Mode 0 01=Mode1 1x=Mode 2 |  | Port A 1= input 0=output | Port C (Upper) 1= input 0=output | Group B Mode Selection 0=Mode 0 1=Mode1 | Port B 1= input 0=output | Port C (Lower) 1= input 0=output |

Bangladesh University of Engineering and Technology

Department of Computer Science and Engineering

CSE 316N

Microprocessors and Microcontrollers Sessional

Experiment 1B

Hand-shake data transmission and reception through 8255.

GOAL:

Understanding the way CPU sends and receives data through handshaking from other devices to control them.

EXPERIMENTAL TOOLS AND MATERIALS: MTS-88.C and I/O BOARD -01

EXPLANATION OF PRINCIPLES:

8255 mode 1 will be used to do input-output. MTS-88.C system board has an 8255 Programmable Peripheral Interfacing (PPI) chip connected with the microprocessor 8088 at address 000100xx b.  DIP switches of the I/O Board are connected with Port A and LEDs are connected with Port B of 8255. Strobe signal is connected to Port C pin PC5.

PROCEDURE:

- Connect I/O Board-01 with the system board by 40 pins flat cable.

- Configure 8255 Port A as input port and Port B as output port.

- Write an 8088 assembly program that will read the status of the DIP switches after getting the strobe signal, manipulate the info as instructed during lab class by the lab instructors and glow the LEDs accordingly.

### CONTROL WORD OF 8255:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode Set Bit 1 | Group A Mode Selection 00=Mode 0 01=Mode1 1x=Mode 2 |  | Port A 1= input 0=output | Port C (Upper) 1= input 0=output | Group B Mode Selection 0=Mode 0 1=Mode1 | Port B 1= input 0=output | Port C (Lower) 1= input 0=output |