Bangladesh University of Engineering and Technology

Department of Computer Science and Engineering

CSE 316

Microprocessors and Microcontrollers Sessional

Experiment 2

Detecting the number of pulses and displaying them.

GOAL:

Use the CPU to carry out the counting function.

EXPERIMENTAL TOOLS AND MATERIALS: MTS-88.C and I/O BOARD -02

EXPLANATION OF PRINCIPLES:

8255 mode 0 will be used to do input-output. MTS-88.C system board has an 8255 (PPI) connected with the microprocessor 8088 at address 000100xx b. PA0 is connected to the push button. PA0 will be ‘1’ when push-button has not been pressed and will be ‘0’ when push-button has been pressed. Two 7-segment displays are connected to Port B through 7448 BCD to 7-segment decoders. PB0-3 controls right display whereas PB4-7 controls left display.

PROCEDURE:

- Connect I/O Board-02 with the system board by 40 pins flat cable.

- Configure 8255 Port A as input port and Port B as output port, both in mode 0.

- Write an 8088 assembly program that will count your pressing of the push-button and will display the count in the 7-segment displays in the format specified by the teachers during the lab.

### CONTROL WORD OF 8255:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode Set Bit 1 | Group A Mode Selection 00=Mode 0 01=Mode1 1x=Mode 2 |  | Port A 1= input 0=output | Port C (Upper) 1= input 0=output | Group B Mode Selection 0=Mode 0 1=Mode1 | Port B 1= input 0=output | Port C (Lower) 1= input 0=output |