Bangladesh University of Engineering and Technology

Department of Computer Science and Engineering

CSE 316

Microprocessors and Microcontrollers Sessional

Experiment 3

Basic LED matrix control, flash, and shift through 8255.

GOAL:

To understand basic LED matrix control in order to generate different characters of user design and data transmission.

EXPERIMENTAL TOOLS AND MATERIALS: MTS-88.C and I/O BOARD -03

EXPLANATION OF PRINCIPLES:

8255 mode 0 will be used to do input-output. MTS-88.C system board has an 8255 (PPI) connected with the microprocessor 8088 at address 000100xx b.

LED Matrix columns are connected with Port B, PB7 being the leftmost column and PB0 being the rightmost column, whereas rows are connected with Port A, PA7 being the topmost row and PA0 being the bottom-most row. To glow a LED corresponding row and column should be simultaneously at “1”.

PROCEDURE:

- Connect I/O Board-03 with the system board by 40 pins flat cable.

- Configure both Port A and Port B of 8255 for output.

- Write an 8088 assembly program that will manipulate the flash and shifts in the direction specified by the teacher in the lab of a pattern specified by the teacher in the LED matrix.

### CONTROL WORD OF 8255:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mode Set Bit 1 | Group A Mode Selection 00=Mode 0 01=Mode1 1x=Mode 2 |  | Port A 1= input 0=output | Port C (Upper) 1= input 0=output | Group B Mode Selection 0=Mode 0 1=Mode1 | Port B 1= input 0=output | Port C (Lower) 1= input 0=output |